# ONFI Technical Errata

| Errata ID | 002 |
| --- | --- |
| Affected Spec Ver. | 5.1 |
| Corrected Spec Ver. |  |

# Submission info

| Name | Company | Date |
| --- | --- | --- |
| John Samson | Micron | 05/24/22 |

To reduce overhead associated with Set/Get Feature commands, NAND vendors may opt to eliminate the busy times related to such sequences. This erratum allows NAND vendors to eliminate traditional tFEAT busy times, replacing them with vendor specific fixed delays following Set/Get Feature commands.

Description of the specification change :

5.29. Set Features Definition

NOTE: NAND vendors may opt to keep the busy signal high, requiring a vendor specific fixed delay time instead. Refer to vendor device datasheet.

Figure 5-4 Set Features timing

5.30. Get Features Definition

NOTE: NAND vendors may opt to keep the busy signal high, requiring a vendor specific fixed delay time instead. Refer to vendor device datasheet.

Figure 5-46 Get Features timing

Disposition log

Technical input submitted to the ONFI Workgroup is subject to the terms of the ONFI contributor’s agreement.