# ONFI Technical Errata

| Errata ID | 005 |
| --- | --- |
| Affected Spec Ver. | 5.0 and 5.1 |
| Corrected Spec Ver. |  |

# Submission info

| Name | Company | Date |
| --- | --- | --- |
| John Samson | Micron | 05/24/22 |

For efficiency improvement of multi-plane read, program and erase sequences, NAND vendors may opt to eliminate the busy times related to such sequences.  This erratum allows NAND vendors to eliminate tPLEBSY during multi-plane erase operation, tPLPBSY during multi-plane program operation and tPLRBSY during multi-plane read operation and instead require vendor specific fixed delays in lieu of these busy times.

Description of the specification change :

4.18.1. General Parameters

| Parameter | Description |
| --- | --- |
| tBERS1 | Block erase time |
| tCCS | Change Column setup time |
| tPLEBSY1, 4 | Busy time for multi-plane erase operation |
| tPLPBSY1, 4 | Busy time for multi-plane program operation |
| tPLRBSY1, 4 | Busy time for multi-plane read operation |
| tPCBSY | Program cache busy time |
| tPROG1 | Page program time |
| tR1 | Page read time |
| tRCBSY1 | Read cache busy time |
| NOTE: - Measured from the falling edge of SR[6] to the rising edge of SR[6]. - For the NV-DDR2, NV-DDR3 and NV-LPDDR4 interfaces, during data input sequences which require tCCS, tCCS is referenced from WE# high to: - The first byte (DQS_t rising edge) input when warmup cycles are disabled - The first byte (DQS_t rising edge) of the first input warmup cycle when warmup cycles are enabled - For the NV-DDR2, NV-DDR3 and NV-LPDDR4 interfaces, during data output sequences which require tCCS, tCCS is referenced from WE# high to the RE_t falling edge marking the start of the read pre-amble (tRPRE/tRPRE2). - NAND vendors may remove tPLEBSY, tPLPBSY and tPLRBSY busy times, keeping SR[6] HIGH between multi-plane sequences, and instead require the host to provide a vendor specific fixed delay between multi-plane sequences (see vendor datasheet). |  |

Table 4-64	Array Timing Parameter Descriptions

4.18.1. General Parameters

| Parameter | Typical | Maximum |
| --- | --- | --- |
| tPLEBSY2 | 500 ns | tBERS |
| tPLPBSY2 | 500 ns | tPROG |
| tPLRBSY2 | 500 ns | tR |
| tPCBSY | 3 µs | tPROG |
| tRCBSY | 3 µs | tR |
| NOTE: - Typical times for tPCBSY and tRCBSY are the recommended interval at which the host should consider polling status. Device busy time may be longer than the typical value. - NAND vendors may remove tPLEBSY, tPLPBSY and tPLRBSY busy times, keeping SR[6] HIGH between multi-plane sequences, and instead require the host to provide a vendor specific fixed delay between multi-plane sequences (see vendor datasheet). |  |  |

Table 4-65	Cache and Multi-plane Short Busy Times

4.18.1. General Parameters

Figure 5-32	Small data moves, data modification

NOTE:

- NAND vendors may remove tPLPBSY busy time, keeping SR[6] HIGH, and instead require the host to provide a vendor

specific fixed delay between the 11h command and the next command (see vendor datasheet).

Disposition log

Technical input submitted to the ONFI Workgroup is subject to the terms of the ONFI contributor’s agreement.