# ONFI Technical Errata

| Errata ID | 007 |
| --- | --- |
| Affected Spec Ver. | 5.0/5.1 |
| Corrected Spec Ver. |  |

# Submission info

| Name | Company | Date |
| --- | --- | --- |
| David Chen | YMTC | 06/22/22 |

ONFI 5.0 table 4-54 / ONFI 5.1 Table 4-56 “Rtt effective impedances, NV-LPDDR4 with ZQ calibration” define the RTT value based on VOH=Vccq/3 option

While NV-LPDDR4 have VOH=Vccq/3 and option for VOH=Vccq/2.5, ZQ calibration is done at corresponding VOH for each option. This erratum adds a note for 0.4xVccQ to the tables.

| Description of the specification change : Rtt | VOUT to VssQ | Maximum | Nominal | Minimum | Unit |
| --- | --- | --- | --- | --- | --- |
| 25 Ohm | 0.1 x VccQ | 1.15 | 1.0 | 0.75 | RZQ/12 |
|  | 0.33 x VccQ4 | 1.15 | 1.0 | 0.85 |  |
|  | 0.5 x VccQ | 1.35 | 1.0 | 0.85 |  |
| 37.5 Ohm | 0.1 x VccQ | 1.15 | 1.0 | 0.75 | RZQ/8 |
|  | 0.33 x VccQ4 | 1.15 | 1.0 | 0.85 |  |
|  | 0.5 x VccQ | 1.35 | 1.0 | 0.85 |  |
| 42.9 Ohm | 0.1 x VccQ | 1.15 | 1.0 | 0.75 | RZQ/7 |
|  | 0.33 x VccQ4 | 1.15 | 1.0 | 0.85 |  |
|  | 0.5 x VccQ | 1.35 | 1.0 | 0.85 |  |
| 50 Ohm | 0.1 x VccQ | 1.15 | 1.0 | 0.75 | RZQ/6 |
|  | 0.33 x VccQ4 | 1.15 | 1.0 | 0.85 |  |
|  | 0.5 x VccQ | 1.35 | 1.0 | 0.85 |  |
| 60 Ohm | 0.1 x VccQ | 1.15 | 1.0 | 0.75 | RZQ/5 |
|  | 0.33 x VccQ4 | 1.15 | 1.0 | 0.85 |  |
|  | 0.5 x VccQ | 1.35 | 1.0 | 0.85 |  |
| 75 Ohm | 0.1 x VccQ | 1.15 | 1.0 | 0.75 | RZQ/4 |
|  | 0.33 x VccQ4 | 1.15 | 1.0 | 0.85 |  |
|  | 0.5 x VccQ | 1.35 | 1.0 | 0.85 |  |
| 100 Ohm | 0.1 x VccQ | 1.15 | 1.0 | 0.75 | RZQ/3 |
|  | 0.33 x VccQ4 | 1.15 | 1.0 | 0.85 |  |
|  | 0.5 x VccQ | 1.35 | 1.0 | 0.85 |  |
| 150 Ohm | 0.1 x VccQ | 1.15 | 1.0 | 0.75 | RZQ/2 |
|  | 0.33 x VccQ4 | 1.15 | 1.0 | 0.85 |  |
|  | 0.5 x VccQ | 1.35 | 1.0 | 0.85 |  |
| 300 Ohm | 0.1 x VccQ | 1.15 | 1.0 | 0.75 | RZQ/1 |
|  | 0.33 x VccQ4 | 1.15 | 1.0 | 0.85 |  |
|  | 0.5 x VccQ | 1.35 | 1.0 | 0.85 |  |
| NOTE: - Tolerance limits assume RZQ of 300Ω +/- 1% and are applicable after proper ZQ calibration has been performed at a stable temperature and voltage. - Refer to ODT Sensitivity if either the temperature or the voltage changes after calibration. - The minimum values are derated by 6% when the device operates between –40°C and 0°C (TC). - For VOH.LTT,nom=VccQ/2.5 setting use 0.4 x VccQ |  |  |  |  |  |

Table 4-54	Rtt effective impedances, NV-LPDDR4 with ZQ calibration

Same change applies to Table 4-56 in ONFI 5.1

Disposition log

Technical input submitted to the ONFI Workgroup is subject to the terms of the ONFI contributor’s agreement.