# ONFI Technical Errata

| Errata ID | 008 |
| --- | --- |
| Affected Spec Ver. | 5.0/5.1 |
| Corrected Spec Ver. |  |

# Submission info

| Name | Company | Date |
| --- | --- | --- |
| Kai-Uwe Schmidt | Solidigm | 07/08/22 |

ONFI 5.0 table 2-17 “NV-DDR3 Differential AC and DC Input Levels” title implies that it only applies to NV-DDR3, when it should also apply to NV-DDR2. Furthermore, a definition for NV-LPDDR4 is missing in that table. The related section 4.14 also misses a differential slew rate measurement definition for NV-LPDDR4.

This erratum changes the title for Table 2-17 to “NV-DDR2/NV-DDR3/NV-LPDDR4 Differential AC and DC Input Levels” and adds a definition for differential DC and AC levels for NV-LPDDR4. In section 4.14.3 Table 4-48, a note is added for differential slew rate calculation.

| Description of the specification change : Parameter | Symbol | Min | Max | Units |
| --- | --- | --- | --- | --- |
| Differential input high | VIHdiff (DC) | 2 x [VIH.SSTL (DC) – VREFQ] 2 2 x [VIH.LTT (DC) – Vcent] 3 | Refer to Note 1. | V |
| Differential input low | VILdiff (DC) | Refer to Note 1. | 2 x [VIL.SSTL (DC) – VREFQ] 2 2 x [VIL.LTT (DC) – Vcent] 3 | V |
| Differential input high AC | VIHdiff (AC) | 2 x [VIH.SSTL (AC) – VREFQ] 2 2 x [VIH.LTT (AC) – Vcent] 3 | Refer to Note 1. | V |
| Differential input low AC | VILdiff (AC) | Refer to Note 1. | 2 x [VIL.SSTL (AC) – VREFQ] 2 x [VIL.LTT (AC) – Vcent] 3 | V |
| NOTE: - These values are not defined. However, the single-ended signals (RE_t, RE_c, DQS_t, and DQS_c) need to be within the respective limits [VIH(DC) max, VIL (DC) min] for single-ended signals as well as the limitations for overshoot and undershoot in Table 2-13, Table 2-14 and Table 2-15. - Applies to NV-DDR2/3 - Applies to NV-LPDDR4 |  |  |  |  |

Table 2-17	NV-DDR2/NV-DDR3/NV-LPDDR4 Differential AC and DC Input Levels

| Description | Single Ended | Differential | Unit |
| --- | --- | --- | --- |
| Input slew rate (min) | 1.0 | 2.0 | V/ns |
| Input slew rate (max) | 4.5 | 9.0 | V/ns |
| Notes: - Single Ended Input Slew Rate = (Vcent_DQ – VIL.LTT(DC)) / ∆TF and (VIH.LTT(DC) – Vcent_DQ) / ∆TR - Differential Input Slew Rate = (VIH.LTT(DC) – VIL.LTT(DC)) / ∆TF and (VIH.LTT(DC) – VIL.LTT(DC)) / ∆TR |  |  |  |

Table 4-48	NV-LPDDR4 Maximum and Minimum Input Slew Rate

Disposition log

Technical input submitted to the ONFI Workgroup is subject to the terms of the ONFI contributor’s agreement.