# ONFI Technical Errata

| Errata ID | 010 |
| --- | --- |
| Affected Spec Ver. | 5.1, 5.0 |
| Corrected Spec Ver. |  |

# Submission info

| Name | Company | Date |
| --- | --- | --- |
| Rey de Luna | Micron | 10/10/22 |

To support faster data rates, NAND vendors may choose technologies and implementations leading to higher NV-DDR3 interface ODT PMOS/NMOS resistance mismatch.

This ECN relaxes NV-DDR3 ODT ΔVm limits from +/-7% to +/- 15%.

# Description of the specification change :

In “On-die Termination DC Electrical Characteristics, NV-DDR3 without ZQ calibration” and “On-die Termination DC Electrical Characteristics, NV-DDR2 and NV-DDR3 with ZQ calibration” tables,  modify ΔVm max from +7% to +15% and ΔVm min from -7% to -15%. The table references and table contents below are taken from the ONFI5.1 specification:

| Parameter | Symbol | Min | Nom | Max | Unit | Optional or Mandatory |
| --- | --- | --- | --- | --- | --- | --- |
| Rtt effective Impedance value for 50 Ohms setting | RttEff1 | 32.5 | 50 | 80 | Ohms | Mandatory |
| Rtt effective impedance value for 75 Ohms setting | RttEff2 | 48.5 | 75 | 120 | Ohms | Mandatory |
| Rtt effective impedance value for 100 Ohms setting | RttEff3 | 65 | 100 | 160 | Ohms | Optional1 |
| Rtt effective impedance value for 150 Ohms setting | RttEff4 | 97.5 | 150 | 240 | Ohms | Mandatory |
| Deviation of VM with respect to VccQ/2 | ∆VM | -7 -15 | - | 7 15 | % |  |
| NOTE: - Devices that support 37.5 Ohms and not 35 Ohms do not support Rtt = 100 Ohms. |  |  |  |  |  |  |

Table 4-53  On-die Termination DC Electrical Characteristics, NV-DDR3 without ZQ calibration

| Parameter | Symbol | Min | Nom | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| Rtt effective Impedance value | RttEff | See Table 4-55 |  |  | Ohms |
| Deviation of VM with respect to VccQ/2 | ∆VM | -7 -15 | - | 7 15 | % |

Table 4-54 On-die Termination DC Electrical Characteristics, NV-DDR2 and NV-DDR3 with ZQ calibration

Disposition log

Technical input submitted to the ONFI Workgroup is subject to the terms of the ONFI contributor’s agreement.