# ONFI Technical Errata

| Errata ID | 011 |
| --- | --- |
| Affected Spec Ver. | 5.0/5.1 |
| Corrected Spec Ver. |  |

# Submission info

| Name | Company | Date |
| --- | --- | --- |
| Jaehyeong Hong Ji-Hye Bae | SK hynix | Oct 11, 2022 |

ONFI 5.1 table 5-15 introduces Feature Address 02h (=FA02h). In FA02h P3, <3:2> bit indicates the range and step size of internal VrefQ settings. However, the detail explanation of each case in FA02h P3 <3:2> does not exist. 3 cases of internal VrefQ range and step size, value 1/value 2/value 3, which are not defined in ONFi 5.1 FA23h. Therefore, the detail explanation for each internal VrefQ range/step size is added in FA23h. In addition, internal VrefQ can be used for NV-DDR3 over 1600Mbps, which changes the 5.31.6 FA 23h interface support range. Also, to align with JEDEC standard, P1 becomes more detail in table 5-21. Furthermore, value1 spec summary is introduced in table 5-22 and previous table 5-22 is changes to table 5-23. Lastly, value2 spec is introduced in table 5-24 and table 5-25 with two notations for proper VrefQ setting.

Description of the specification change :

5.31.6. Internal VrefQ Value – FA 23h

This setting shall be supported if the target supports the NV-DDR3 which provides internal VrefQ function and NV-LPDDR4 data interface. The internal VrefQ value setting shall be retained across Reset (FFh), Synchronous Reset (FCh), and Reset LUN (FAh) commands. The power-on default internal VrefQ value is 35% of VccQ (46h).

| Sub-feature Parameter | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P0 | Internal VrefQ Value1/Value2/Value3 Settings |  |  |  |  |  |  | Reserved |
| P1 | Reserved (0) |  |  |  |  |  |  |  |
| P2 | Reserved (0) |  |  |  |  |  |  |  |
| P3 | Reserved (0) |  |  |  |  |  |  |  |

Table 5-21	Feature Address 23h

Internal VrefQ Value

This field controls the voltage level of the internal VrefQ.

Reserved

Reserved values shall be cleared to zero by the host. Targets shall not be sensitive to the value of reserved fields.

| Internal VrefQ Value1 Range/Step Size |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter | Min | Typ | Max | Unit |
| Default | TBD |  |  | VccQ |
| Vref_min | 0% | - | - | VccQ |
| Vref_max | - | - | 63.50% | VccQ |
| Vref_step | 0.35% | 0.50% | 0.65% | VccQ |
| Vref_Set_Tol | -1.75% | 0% | 1.75% | VccQ |

Table 5-22	Internal VrefQ Value Range/Step-Size/Tolerance for Type1 or Value1 Settings

| Internal VrefQ Code | VrefQ (% of VccQ) | Internal VrefQ Code | VrefQ (% of VccQ) | Internal VrefQ Code | VrefQ (% of VccQ) | Internal VrefQ Code | VrefQ (% of VccQ) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 00h | 0.00 | 20h | 16.00 | 40h | 32.00 | 60h | 48.00 |
| 01h | 0.50 | 21h | 16.50 | 41h | 32.50 | 61h | 48.50 |
| 02h | 1.00 | 22h | 17.00 | 42h | 33.00 | 62h | 49.00 |
| 03h | 1.50 | 23h | 17.50 | 43h | 33.50 | 63h | 49.50 |
| 04h | 2.00 | 24h | 18.00 | 44h | 34.00 | 64h | 50.00 |
| 05h | 2.50 | 25h | 18.50 | 45h | 34.50 | 65h | 50.50 |
| 06h | 3.00 | 26h | 19.00 | 46h | 35.00 | 66h | 51.00 |
| 07h | 3.50 | 27h | 19.50 | 47h | 35.50 | 67h | 51.50 |
| 08h | 4.00 | 28h | 20.00 | 48h | 36.00 | 68h | 52.00 |
| 09h | 4.50 | 29h | 20.50 | 49h | 36.50 | 69h | 52.50 |
| 0Ah | 5.00 | 2Ah | 21.00 | 4Ah | 37.00 | 6Ah | 53.00 |
| 0Bh | 5.50 | 2Bh | 21.50 | 4Bh | 37.50 | 6Bh | 53.50 |
| 0Ch | 6.00 | 2Ch | 22.00 | 4Ch | 38.00 | 6Ch | 54.00 |
| 0Dh | 6.50 | 2Dh | 22.50 | 4Dh | 38.50 | 6Dh | 54.50 |
| 0Eh | 7.00 | 2Eh | 23.00 | 4Eh | 39.00 | 6Eh | 55.00 |
| 0Fh | 7.50 | 2Fh | 23.50 | 4Fh | 39.50 | 6Fh | 55.50 |
| 10h | 8.00 | 30h | 24.00 | 50h | 40.00 | 70h | 56.00 |
| 11h | 8.50 | 31h | 24.50 | 51h | 40.50 | 71h | 56.50 |
| 12h | 9.00 | 32h | 25.00 | 52h | 41.00 | 72h | 57.00 |
| 13h | 9.50 | 33h | 25.50 | 53h | 41.50 | 73h | 57.50 |
| 14h | 10.00 | 34h | 26.00 | 54h | 42.00 | 74h | 58.00 |
| 15h | 10.50 | 35h | 26.50 | 55h | 42.50 | 75h | 58.50 |
| 16h | 11.00 | 36h | 27.00 | 56h | 43.00 | 76h | 59.00 |
| 17h | 11.50 | 37h | 27.50 | 57h | 43.50 | 77h | 59.50 |
| 18h | 12.00 | 38h | 28.00 | 58h | 44.00 | 78h | 60.00 |
| 19h | 12.50 | 39h | 28.50 | 59h | 44.50 | 79h | 60.50 |
| 1Ah | 13.00 | 3Ah | 29.00 | 5Ah | 45.00 | 7Ah | 61.00 |
| 1Bh | 13.50 | 3Bh | 29.50 | 5Bh | 45.50 | 7Bh | 61.50 |
| 1Ch | 14.00 | 3Ch | 30.00 | 5Ch | 46.00 | 7Ch | 62.00 |
| 1Dh | 14.50 | 3Dh | 30.50 | 5Dh | 46.50 | 7Dh | 62.50 |
| 1Eh | 15.00 | 3Eh | 31.00 | 5Eh | 47.00 | 7Eh | 63.00 |
| 1Fh | 15.50 | 3Fh | 31.50 | 5Fh | 47.50 | 7Fh | 63.50 |

Table 5-23	Internal VrefQ Value1 Setting Versus Value as % of VccQ

| Internal VrefQ Value2 Range/Step Size |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter | Min | Typ | Max | Unit |
| Default | TBD |  |  | VccQ |
| Vref_min | 0% | - | - | VccQ |
| Vref_max | - | - | 99.22% | VccQ |
| Vref_step | 0.58% | 0.78% | 0.98% | VccQ |
| Vref_Set_Tol | -1.95% | 0% | 1.95% | VccQ |

Table 5-24	Internal VrefQ Value Range/Step-Size/Tolerance for Type2 or Value2 Settings

| Internal VrefQ Code | VrefQ (% of VccQ) | Internal VrefQ Code | VrefQ (% of VccQ) | Internal VrefQ Code | VrefQ (% of VccQ) | Internal VrefQ Code | VrefQ (% of VccQ) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 00h | 0.00 | 20h | 25.00 | 40h | 50.00 | 60h | 75.00 |
| 01h | 0.78 | 21h | 25.78 | 41h | 50.78 | 61h | 75.78 |
| 02h | 1.56 | 22h | 26.56 | 42h | 51.56 | 62h | 76.56 |
| 03h | 2.34 | 23h | 27.34 | 43h | 52.34 | 63h | 77.34 |
| 04h | 3.13 | 24h | 28.13 | 44h | 53.13 | 64h | 78.13 |
| 05h | 3.91 | 25h | 28.91 | 45h | 53.91 | 65h | 78.91 |
| 06h | 4.69 | 26h | 29.69 | 46h | 54.69 | 66h | 79.69 |
| 07h | 5.47 | 27h | 30.47 | 47h | 55.47 | 67h | 80.47 |
| 08h | 6.25 | 28h | 31.25 | 48h | 56.25 | 68h | 81.25 |
| 09h | 7.03 | 29h | 32.03 | 49h | 57.03 | 69h | 82.03 |
| 0Ah | 7.81 | 2Ah | 32.81 | 4Ah | 57.81 | 6Ah | 82.81 |
| 0Bh | 8.59 | 2Bh | 33.59 | 4Bh | 58.59 | 6Bh | 83.59 |
| 0Ch | 9.38 | 2Ch | 34.38 | 4Ch | 59.38 | 6Ch | 84.38 |
| 0Dh | 10.16 | 2Dh | 35.16 | 4Dh | 60.16 | 6Dh | 85.16 |
| 0Eh | 10.94 | 2Eh | 35.94 | 4Eh | 60.94 | 6Eh | 85.94 |
| 0Fh | 11.72 | 2Fh | 36.72 | 4Fh | 61.72 | 6Fh | 86.72 |
| 10h | 12.50 | 30h | 37.50 | 50h | 62.50 | 70h | 87.50 |
| 11h | 13.28 | 31h | 38.28 | 51h | 63.28 | 71h | 88.28 |
| 12h | 14.06 | 32h | 39.06 | 52h | 64.06 | 72h | 89.06 |
| 13h | 14.84 | 33h | 39.84 | 53h | 64.84 | 73h | 89.84 |
| 14h | 15.63 | 34h | 40.63 | 54h | 65.63 | 74h | 90.63 |
| 15h | 16.41 | 35h | 41.41 | 55h | 66.41 | 75h | 91.41 |
| 16h | 17.19 | 36h | 42.19 | 56h | 67.19 | 76h | 92.19 |
| 17h | 17.97 | 37h | 42.97 | 57h | 67.97 | 77h | 92.97 |
| 18h | 18.75 | 38h | 43.75 | 58h | 68.75 | 78h | 93.75 |
| 19h | 19.53 | 39h | 44.53 | 59h | 69.53 | 79h | 94.53 |
| 1Ah | 20.31 | 3Ah | 45.31 | 5Ah | 70.31 | 7Ah | 95.31 |
| 1Bh | 21.09 | 3Bh | 46.09 | 5Bh | 71.09 | 7Bh | 96.09 |
| 1Ch | 21.88 | 3Ch | 46.88 | 5Ch | 71.88 | 7Ch | 96.88 |
| 1Dh | 22.66 | 3Dh | 47.66 | 5Dh | 72.66 | 7Dh | 97.66 |
| 1Eh | 23.44 | 3Eh | 48.44 | 5Eh | 73.44 | 7Eh | 98.44 |
| 1Fh | 24.22 | 3Fh | 49.22 | 5Fh | 74.22 | 7Fh | 99.22 |

Table 5-25	Internal VrefQ Value2 Setting Versus Value as % of VccQ

NOTE  1    The NAND internal VrefQ shall not be set to a setting beyond the allowable range even during Write Training.

NOTE  2    NAND is not required to implement VrefQ beyond the NAND Minimum Internal VrefQ Allowable Range.

Disposition log

Technical input submitted to the ONFI Workgroup is subject to the terms of the ONFI contributor’s agreement.