# ONFI Technical Errata

| Errata ID | 012 |
| --- | --- |
| Affected Spec Ver. | 5.1 |
| Corrected Spec Ver. | 5.2 |

# Submission info

| Name | Company | Date |
| --- | --- | --- |
| Rey de Luna | Micron | 05/30/23 |

This ECN relaxes ICC1 and ICC2 for devices that support >1600MT/s.

# Description of the specification change :

In Table 2-10,  modify ICC1 and ICC2 Max Spec values as shown below

| Parameter | Symbol | Test Conditions | Min | Typ | Max | Units |
| --- | --- | --- | --- | --- | --- | --- |
| Array read current | ICC1 | Refer to Appendix D | - | - | 100 /150/2007 | mA |
| Array program current | ICC2 |  | - | - | 100 /150/2007 | mA |
| Array erase current | ICC3 |  | - | - | 100 | mA |
| I/O burst read current | ICC4R |  | - | - | 50/100/135/1805 | mA |
|  | ICCQ4R4 |  | - | - | 50/100/135/1805 | mA |
| I/O burst write current | ICC4W |  | - | - | 50/100/135/1805 | mA |
|  | ICCQ4W |  | - | - | 50/100/135/1805 | mA |
| Bus idle current | ICC5 |  | - | - | 15 | mA |
| Standby current | ISB | CE_n=VccQ-0.2V, WP_n=0V/VccQ | - | - | 100 | µA |
| Staggered power-up current | IST1 | CE_n=VccQ-0.2V tRise = 1 ms cLine = 0.1 µF | - | - | 10 | mA |
| Vpp Idle current | IPPI6 | - | - | - | 10 | uA |
| Vpp Active current | IPPA6 | - | - | - | 5 | mA |
| NOTE: - Refer to Appendix C for an exception to the IST current requirement. - ICC1, ICC2, and ICC3 as listed in this table are active current values. For details on how to calculate the active current from the measured values, refer to Appendix D. - During cache operations, increased ICC current is allowed while data is being transferred on the bus and an array operation is ongoing. For a cached read this value is ICC1 + ICC4R; for a cached write this value is ICC2(active) + ICC4W. - For ICCQ4R the test conditions in Appendix D specify IOUT = 0 mA. When Iout is not equal to 0 mA, additional VccQ switching current will be drawn that is highly dependent on system configuration. IccQ due to loading without IOUT = 0 mA may be calculated for each output pin assuming 50% data switching as (IccQ = 0.5 * CL * VccQ * frequency), where CL is the capacitive load. - When the data frequency is above 800 MT/s, then the LUN may consume 180 mA. When the data frequency is above 400MT/s and below or equal to 800 MT/s, then the LUN may consume 135 mA. When the data frequency is above 200 MT/s and below or equal to 400 MT/s, then the LUN may consume 100 mA. When the data frequency is below or equal to 200 MT/s, then the LUN may consume 50 mA. - IPP Idle current is IPP current measured when Vpp is supplied and Vpp is not enabled via Set Feature. IPP Active current is IPP current measured when Vpp is supplied and Vpp is enabled via Set Feature. - When the data frequency is above 2400 MT/s, then the LUN may consume 200 mA. When the data frequency is above 1600 MT/s, then the LUN may consume 150 mA. When the data frequency is below or equal to 1600 MT/s, then the LUN may consume 100 mA. |  |  |  |  |  |  |

Disposition log

Technical input submitted to the ONFI Workgroup is subject to the terms of the ONFI contributor’s agreement.