# ONFI Technical Errata

| Errata ID | 013 |
| --- | --- |
| Affected Spec Ver. | 5.0/5.1 |
| Corrected Spec Ver. | 5.2 |

# Submission info

| Name | Company | Date |
| --- | --- | --- |
| Sriram Balasubrahmanyam | Intel Corporation | May 30, 2023 |

In Table 2-14 “DC and Operating Conditions for VccQ of 1.2V (NV-DDR3), measured on VccQ rail”, for Timing mode 16 – 21 & Table 2-15 Controller DC RX Mask Conditions for VccQ of 1.2V (NV-DDR3), few disconnects

- VIH.SSTL/VIL.SSTL (DC) is set to VREFQ±80mV whereas VDIVW.SSTL which is the DQ RX Mask Voltage total is set to 180mVwhich does not align (20mV higher than VIH/VIL DC)

- VIH.SSTL/VIL.SSTL (AC) is set to VREFQ±100mV whereas VIHL.SSTL (AC) which is the DQ AC Input pulse amplitude pk-pk set to 220mVwhich does not align (20mV higher than VIH/VIL AC)

Other Timing modes as well as LPDDR4 Table 2-16/2-17 have them aligned between VIH/VIL and RX Mask Voltage levels

The ECN is proposing to fix the Table (2-14) to align with JEDEC JESD 230F updated Ballot Motion

- Change VIH.SSTL/VIL.SSTL (DC) for Timing mode 16-21 from ±80mV to ±90mV

- Change VIH.SSTL/VIL.SSTL (AC) for Timing mode 16-21 from ±100mV to ±110mV

Description of the specification change :

ONFI5.1 Spec Table 2-14:  DC and Operating Conditions for VccQ of 1.2V (NV-DDR3), measured on VccQ rail

| Parameter | Symbol | Test Conditions | Min | Typ | Max | Units |
| --- | --- | --- | --- | --- | --- | --- |
| DC Input high voltage w/ VREFQ9 | VIH.SSTL4,7 (DC) | Timing mode 0 – 15 | VREFQ + 100 | - | VccQ | mV |
|  |  | Timing mode 16 - 21 | VREFQ + 80 VREFQ + 90 |  |  |  |
|  |  | Timing mode 22 | VREFQ + 70 |  |  |  |
| AC Input high voltage w/ VREFQ9 | VIH.SSTL4,7 (AC) | Timing mode 0 – 15 | VREFQ + 150 | - | (Note 1) | mV |
|  |  | Timing mode 16 – 21 | VREFQ + 100 VREFQ + 110 |  |  |  |
|  |  | Timing mode 22 | VREFQ + 95 |  |  |  |
| DC Input low voltage w/ VREFQ9 | VIL.SSTL4,7 (DC) | Timing mode 0 – 15 | VssQ | - | VREFQ - 100 | mV |
|  |  | Timing mode 16 - 21 |  |  | VREFQ – 80 VREFQ - 90 |  |
|  |  | Timing mode 22 |  |  | VREFQ - 70 |  |
| AC Input low voltage w/ VREFQ9 | VIL.SSTL4,7 (AC) | Timing mode 0 – 15 | (Note 1) | - | VREFQ - 150 | mV |
|  |  | Timing mode 16 – 21 |  |  | VREFQ – 100 VREFQ - 110 |  |
|  |  | Timing mode 22 |  |  | VREFQ - 95 |  |

Disposition log

Technical input submitted to the ONFI Workgroup is subject to the terms of the ONFI contributor’s agreement.