# ONFI Technical Errata

| Errata ID | 014 |
| --- | --- |
| Affected Spec Ver. | 5.1, 5.0, 4.2, 4.1, 4.0 |
| Corrected Spec Ver. |  |

# Submission info

| Name | Company | Date |
| --- | --- | --- |
| Rey de Luna | Micron | 11/07/2023 |

This erratum intends to clarify the NV-DDR2/NV-DDR3/NV-LPDDR4 bus-state table (4.3.3 in ONFI5.1), tWPSTH timing w.r.t WE# during data burst input with confirm command and includes additional Data Input Burst and Output Burst timing diagrams & notes.

- The bus-state table clarifications

  - Existing table has DQS states as “undefined/(-)” during command and address cycles which created confusion between different burst-exit, confirm command scenarios. This errata clarifies DQS state is recommended to be ‘1’ during command cycles except in case where a data input burst is terminated with CLE=1 with a confirm command where DQS must be held Low to meet tWPSTH and tCDQSH timing requirements. DQS is also recommended to be 1 during address cycles. The Errata also clarifies the table with a note

  - Added a Note-6 at the bottom of Burst-state Table “DQS is recommended to be 1 during command cycles, except in the case where a data input burst is terminated with a CLE=1 and a confirm command is issued, where DQS must be held LOW to meet tWPSTH and tCDQSH requirements. DQS is also recommended to be 1 during address cycles.”

  - Existing Table has WE_n has “-“ during command/address cycle. Added a clock pictorial showing Rising edge of WE# latching command/address

  - Existing Table has DQS_t has “-“ during Data Input/Output cycles. Added a clock pictorial showing Rising/falling edge of DQS_t latching for data

  - Existing Table has RE_t has “-“ during Data Output cycles. Added a clock pictorial showing Rising/falling edge of RE_t latching for data

- For Data Input burst with confirm command(s), after the burst exit with CLE going High (1), Timing diagrams show that WE# should toggle low after tWPSTH. But, tWPSTH is defined as “DQS write postamble hold time” and is not defined w.r.t WE# even though all vendors follow this timing. This errata adds a note:3 to  tWPSTH spec as “For Data-input burst timing followed by a confirm command, at the end of burst with CLE going HIGH, WE# must toggle LOW only after meeting tWPSTH (min) timing”. Example shows the need for tDBS when confirm command cycle is followed by Idle. tDBS ensures that there are no DQS/ODT glitches at the start of Idle (when CLE goes LOW)

- The data input burst exit is confusing with no clarity on tDBS behavior at the end of the burst. Added a clarifying diagram/example for “CLE Burst-end with Confirm Command, tDBS before CLE de-asserted”. Also added a note “DQS is required to satisfy tDBS spec requirement only when confirm command cycle is immediately followed by Idle state”

- Added additional Timing Diagrams for Data Output burst end scenarios as examples

  - CE# Burst-End, Keep CE# de-asserted

  - CLE Burst-end followed by command cycle

# Description of the specification change :

Errata:1  The Bus-state Table clarifications for NV-DDR2, NV-DDR3, NV-LPDDR4 for Table 4-5 in ONFI5.1 spec

Changes 1a, 1b, 1c, 1d, 1e aligned with Change description in the errata

Updated Table 4-5

Errata:2  For Data Input burst with confirm command(s), after the burst exit with CLE going High (1) Timing diagrams show that WE# should toggle low after tWPSTH. But, tWPSTH is defined as “DQS write postamble hold time” and is not defined w.r.t WE# even though all vendors follow this timing.

The ONFI5.1 Table 4-70 shows the tWPSTH definition (& previous ONFI versions also is same) where it defines tWPSTH only w.r.t DQS

The Data Input burst diagram with confirm command below indicates “tWPSTH” as CLE going high to WE# going low

Updated Table 4-70 (ONFI5.1) with Note: 3 on tWPSTH spec

Errata:3  Added a clarifying diagram/example for “CLE Burst-end with Confirm Command, tDBS before CLE de-asserted”.

Fig: CLE Burst-end with Confirm Command, tDBS before CLE de-asserted

Note: DQS is required to satisfy tDBS spec requirement only when confirm command cycle is immediately followed by Idle state

Errata:4a  Added a clarifying diagram/example for “CE# Burst-End, Keep CE# de-asserted”

Fig: CE# Burst-End, Keep CE# de-asserted

Errata:4b  Added a clarifying diagram/example for “CLE Burst-end followed by command cycle”

Fig: CLE Burst-end followed by command cycle

Disposition log

Technical input submitted to the ONFI Workgroup is subject to the terms of the ONFI contributor’s agreement.