# ONFI Technical Errata

| Errata ID | 015 |
| --- | --- |
| Affected Spec Ver. | 5.0, 5.1 |
| Corrected Spec Ver. |  |

# Submission info

| Name | Company | Date |
| --- | --- | --- |
| Rey de Luna | Micron | 11/21/23 |

As NAND device performance increases, the IPPA limits need to be adjusted to accommodate higher performing NAND.  This ECN relaxes the IPPA per LUN limit in the spec.

# Description of the specification change :

In Table 2-8 in ONFI5.1 and ONFI5.0 specifications, remove note with Ipp limit per LUN as shown below:

| Parameter | Symbol | Min | Typ | Max | Units |
| --- | --- | --- | --- | --- | --- |
| Supply voltage for 3.3V devices4 3 | VCC | 2.7 | 3.3 | 3.6 | V |
| Supply voltage for 2.5V devices4 3 | VCC | 2.35 | 2.5 | 2.75 | V |
| Supply voltage for 1.8V devices4 3 | VCC | 1.7 | 1.8 | 1.95 | V |
| Supply voltage for 3.3V I/O signaling1,4 3 | VCCQ | 2.7 | 3.3 | 3.6 | V |
| Supply voltage for 1.8V I/O signaling2, 4 3 | VCCQ | 1.7 | 1.8 | 1.95 | V |
| Supply voltage for 1.2V I/O signaling4 3 | VCCQ | 1.14 | 1.2 | 1.26 | V |
| Ground voltage supply | VSS | 0 | 0 | 0 | V |
| Ground voltage supply for I/O signaling | VSSQ | 0 | 0 | 0 | V |
| External voltage supply3 | VPP | 10.8 | 12.0 | 13.2 | V |
| NOTE: 1. 3.3V VccQ is not supported for NV-DDR2. 2. 3.3V and 1.8V VccQ are not supported for NV-DDR3 or NV-LPDDR4. 3. The maximum external voltage supply current (IPP) is 5 mA per LUN. 4. 3. AC Noise on the supply voltages shall not exceed +/- 3%. 10 kHz to 800 MHz AC and DC noise together shall stay within the Min-Max range specified in this table. |  |  |  |  |  |

Table 2-8 Recommended DC operating conditions

# Description of the specification change :

In Table 2-10 in ONFI5.1 and ONFI5.0 specifications, changed IPPA max as shown below and added Vpp to table description:

| Parameter | Symbol | Test Conditions | Min | Typ | Max | Units |
| --- | --- | --- | --- | --- | --- | --- |
| Array read current | ICC1 | Refer to Appendix D | - | - | 100 | mA |
| Array program current | ICC2 |  | - | - | 100 | mA |
| Array erase current | ICC3 |  | - | - | 100 | mA |
| I/O burst read current | ICC4R |  | - | - | 50/100/135/1805 | mA |
|  | ICCQ4R4 |  | - | - | 50/100/135/1805 | mA |
| I/O burst write current | ICC4W |  | - | - | 50/100/135/1805 | mA |
|  | ICCQ4W |  | - | - | 50/100/135/1805 | mA |
| Bus idle current | ICC5 |  | - | - | 15 | mA |
| Standby current | ISB | CE_n=VccQ-0.2V, WP_n=0V/VccQ | - | - | 100 | µA |
| Staggered power-up current | IST1 | CE_n=VccQ-0.2V tRise = 1 ms cLine = 0.1 µF | - | - | 10 | mA |
| Vpp Idle current | IPPI6 | - | - | - | 10 | uA |
| Vpp Active current | IPPA6 | - | - | - | 5 15 | mA |
| NOTE: - Refer to Appendix C for an exception to the IST current requirement. - ICC1, ICC2, and ICC3 as listed in this table are active current values. For details on how to calculate the active current from the measured values, refer to Appendix D. - During cache operations, increased ICC current is allowed while data is being transferred on the bus and an array operation is ongoing. For a cached read this value is ICC1 + ICC4R; for a cached write this value is ICC2(active) + ICC4W. - For ICCQ4R the test conditions in Appendix D specify IOUT = 0 mA. When Iout is not equal to 0 mA, additional VccQ switching current will be drawn that is highly dependent on system configuration. IccQ due to loading without IOUT = 0 mA may be calculated for each output pin assuming 50% data switching as (IccQ = 0.5 * CL * VccQ * frequency), where CL is the capacitive load. - When the data frequency is above 800 MT/s, then the LUN may consume 180 mA. When the data frequency is above 400MT/s and below or equal to 800 MT/s, then the LUN may consume 135 mA. When the data frequency is above 200 MT/s and below or equal to 400 MT/s, then the LUN may consume 100 mA. When the data frequency is below or equal to 200 MT/s, then the LUN may consume 50 mA. - IPP Idle current is IPP current measured when Vpp is supplied and Vpp is not enabled via Set Feature. IPP Active current is IPP current measured when Vpp is supplied and Vpp is enabled via Set Feature. |  |  |  |  |  |  |

Table 2-10 DC and Operating Conditions for raw NAND, measured on Vcc, VccQ or Vpp rail

Disposition log

Technical input submitted to the ONFI Workgroup is subject to the terms of the ONFI contributor’s agreement.