# ONFI Technical Errata

| Errata ID | 016 |
| --- | --- |
| Affected Spec Ver. | 5.1 |
| Corrected Spec Ver. |  |

# Submission info

| Name | Company | Date |
| --- | --- | --- |
| Ji Hye Bae | SK hynix | Jan 16, 2024 |

As NAND device performance increases, the ISB limits need to be adjusted to accommodate higher performing NAND.  This ECN relaxes the ISB per LUN limit in the spec, and corrects verbiage from notes.

# Description of the specification change :

In Table 2-11 in ONFI5.1 specifications, add ISB value and note 8 that states ISB value can be up to 200uA at 2400MT/s or above.

By including verbage “up to” prior to defined maximum current in note 5, note 7 and note 8, this ECN updates Table 2-11 verbages in notes to express Max current is the maximum limit of current that LUN may consume up to.

Technical input submitted to the ONFI Workgroup is subject to the terms of the ONFI contributor’s agreement.